MIDTERM — SECURE ENTERPRISE NETWORK

Complete Packet Tracer Configuration Reference

# 1. IP ADDRESSING PLAN

## 1.1 WAN Link

| Device | Interface | IP Address | Subnet Mask | Notes |
| --- | --- | --- | --- | --- |
| HQ-Router | G0/0 (WAN) | 10.10.10.1 | 255.255.255.252 | /30 — 2 usable hosts |
| Branch-Router | G0/0 (WAN) | 10.10.10.2 | 255.255.255.252 | /30 |

## 1.2 LAN Segments

| LAN | Network | Mask | Gateway (Router) | PC Range | VLAN |
| --- | --- | --- | --- | --- | --- |
| Admin LAN | 192.168.10.0/24 | 255.255.255.0 | 192.168.10.1 | .2 – .254 | VLAN 10 |
| Staff LAN | 192.168.20.0/25 | 255.255.255.128 | 192.168.20.1 | .2 – .126 | VLAN 20 |
| Wireless LAN | 192.168.30.0/26 | 192.168.30.1 | 255.255.255.192 | .2 – .62 | VLAN 30 |

## 1.3 Server

| Device | IP | Subnet Mask | Gateway | Services |
| --- | --- | --- | --- | --- |
| HQ-Server | 192.168.10.100 | 255.255.255.0 | 192.168.10.1 | HTTP, HTTPS, FTP |

# 2. DEVICE SECURITY (All Routers & Switches)

Apply to EVERY router and switch before anything else.

IOS Commands:

enable

configure terminal

hostname HQ-Router          ! change per device

enable secret cicsoenpa55

line console 0

password ciscoconpa55

login

exit

service password-encryption

banner motd # WARNING: Unauthorized access is prohibited. Disconnect now. #

exit

write memory

Hostnames to use: HQ-Router, Branch-Router, HQ-Switch, Branch-Switch

# 3. ROUTER INTERFACE CONFIGURATION

## 3.1 HQ-Router

! WAN interface (connects to Branch-Router)

interface GigabitEthernet0/0

ip address 10.10.10.1 255.255.255.252

no shutdown

! Admin LAN (subinterface for VLAN 10 or direct)

interface GigabitEthernet0/1

ip address 192.168.10.1 255.255.255.0

no shutdown

! Staff LAN

interface GigabitEthernet0/2

ip address 192.168.20.1 255.255.255.128

no shutdown

! Wireless LAN

interface GigabitEthernet0/3

ip address 192.168.30.1 255.255.255.192

no shutdown

## 3.2 Branch-Router

! WAN interface (connects to HQ-Router)

interface GigabitEthernet0/0

ip address 10.10.10.2 255.255.255.252

no shutdown

## 3.3 Static Routing

! On HQ-Router — if Branch has additional LANs:

ip route 0.0.0.0 0.0.0.0 10.10.10.2

! On Branch-Router — route all internal traffic to HQ:

ip route 192.168.10.0 255.255.255.0 10.10.10.1

ip route 192.168.20.0 255.255.255.128 10.10.10.1

ip route 192.168.30.0 255.255.255.192 10.10.10.1

# 4. SWITCH CONFIGURATION

## 4.1 VLAN Setup

enable

configure terminal

vlan 10

name Admin

vlan 20

name Staff

vlan 30

name Wireless

vlan 99

name UNUSED

exit

## 4.2 Port Assignments (HQ-Switch)

| Port | VLAN | Device |
| --- | --- | --- |
| Fa0/1 | VLAN 10 | Admin PC1 |
| Fa0/2 | VLAN 10 | Admin PC2 |
| Fa0/3 | VLAN 20 | Staff PC1 |
| Fa0/4 | VLAN 20 | Staff PC2 |
| Fa0/5 | VLAN 30 | Wireless AP / Laptop |
| Fa0/24 | Trunk | Uplink to HQ-Router |
| Fa0/10-23 | VLAN 99 (disabled) | Unused ports |

! Admin ports

interface FastEthernet0/1

switchport mode access

switchport access vlan 10

switchport port-security

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

no shutdown

interface FastEthernet0/2

switchport mode access

switchport access vlan 10

switchport port-security

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

no shutdown

! Staff ports

interface FastEthernet0/3

switchport mode access

switchport access vlan 20

switchport port-security

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

no shutdown

interface FastEthernet0/4

switchport mode access

switchport access vlan 20

switchport port-security

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

no shutdown

! Wireless port

interface FastEthernet0/5

switchport mode access

switchport access vlan 30

switchport port-security

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

no shutdown

! Uplink trunk to router

interface FastEthernet0/24

switchport mode trunk

no shutdown

! Disable and quarantine unused ports

interface range FastEthernet0/6 - 23

switchport mode access

switchport access vlan 99

shutdown

write memory

# 5. ACCESS CONTROL LISTS (ACLs)

Server is at 192.168.10.100. Apply ACLs on HQ-Router.

## 5.1 ACL Logic Summary

| Source | Destination | Service | Action |
| --- | --- | --- | --- |
| Admin LAN (192.168.10.0/24) | Server FTP (port 21) | FTP | PERMIT |
| Staff LAN (192.168.20.0/25) | Server FTP | FTP | DENY |
| Wireless LAN (192.168.30.0/26) | Server FTP | FTP | DENY |
| Staff LAN | Server HTTP/HTTPS | TCP 80/443 | PERMIT |
| Wireless LAN | Server HTTP/HTTPS | TCP 80/443 | PERMIT |
| Wireless LAN | Admin LAN | ANY | DENY |
| Any external | Internal networks | ANY | DENY |

## 5.2 ACL Commands

ip access-list extended LAN_POLICY

! Admin CAN access FTP

permit tcp 192.168.10.0 0.0.0.255 host 192.168.10.100 eq 21

! Staff CAN access HTTP/HTTPS

permit tcp 192.168.20.0 0.0.0.127 host 192.168.10.100 eq 80

permit tcp 192.168.20.0 0.0.0.127 host 192.168.10.100 eq 443

! Staff CANNOT access FTP

deny tcp 192.168.20.0 0.0.0.127 host 192.168.10.100 eq 21

! Wireless CANNOT access Admin LAN

deny ip 192.168.30.0 0.0.0.63 192.168.10.0 0.0.0.255

! Wireless CAN access HTTP/HTTPS

permit tcp 192.168.30.0 0.0.0.63 host 192.168.10.100 eq 80

permit tcp 192.168.30.0 0.0.0.63 host 192.168.10.100 eq 443

! Wireless CANNOT access FTP

deny tcp 192.168.30.0 0.0.0.63 host 192.168.10.100 eq 21

! Permit all other internal traffic

permit ip 192.168.0.0 0.0.255.255 192.168.0.0 0.0.255.255

! Block external attackers from internal resources

deny ip any 192.168.0.0 0.0.255.255

permit ip any any

exit

! Apply inbound on each LAN-facing interface

interface GigabitEthernet0/1

ip access-group LAN_POLICY in

interface GigabitEthernet0/2

ip access-group LAN_POLICY in

interface GigabitEthernet0/3

ip access-group LAN_POLICY in

write memory

# 6. WIRELESS CONFIGURATION

## 6.1 WPA2 Personal Setup (in Packet Tracer GUI)

In Packet Tracer, place a Wireless Router or Access Point connected to HQ-Switch on Fa0/5 (VLAN 30).

| Setting | Value |
| --- | --- |
| SSID | CompanyWireless |
| Security Mode | WPA2-Personal |
| Passphrase | SecurePass123! |
| IP (AP management) | 192.168.30.1 |
| DHCP range | 192.168.30.2 – .62 |
| Default Gateway | 192.168.30.1 |

Wireless clients connect via SSID. ACL on router blocks them from Admin LAN and FTP (see Section 5).

# 7. DEMONSTRATING PORT SECURITY VIOLATION

To trigger a violation during the exam:

! Step 1 — sticky MAC is already learned on Fa0/1 (Admin PC1)

! Step 2 — connect a SECOND PC to the same port Fa0/1

! Step 3 — the switch will restrict/shutdown the port

! Verify the violation happened:

show port-security interface FastEthernet0/1

! Look for: 'Security Violation Count : 1'

! To recover the port after violation (if shutdown mode):

interface FastEthernet0/1

shutdown

no shutdown

# 8. VERIFICATION COMMANDS

Run these to prove everything works to the examiner.

## 8.1 Routing

show ip route                   ! verify routing table has all subnets

ping 10.10.10.2                 ! HQ-Router pinging Branch-Router WAN

ping 192.168.20.1               ! HQ pinging Staff LAN gateway

ping 192.168.30.1               ! HQ pinging Wireless LAN gateway

## 8.2 VLANs & Switching

show vlan brief                 ! all VLANs and assigned ports

show interfaces trunk           ! verify trunk port

show port-security              ! all ports with security config

show port-security interface Fa0/1

## 8.3 ACLs

show ip access-lists            ! see all ACL rules + hit counts

show running-config | include access-group  ! verify ACLs applied to interfaces

! Test from PC CLI (Desktop > Command Prompt):

! Staff PC pinging server HTTP — should work:

!   ping 192.168.10.100

! Staff PC trying FTP — should fail (no reply):

!   ftp 192.168.10.100

! Wireless PC pinging Admin PC — should fail:

!   ping 192.168.10.2

## 8.4 Device Security

show running-config | include hostname

show running-config | include enable secret

show running-config | include service password

show running-config | include banner

# 9. EXAM SUBMISSION CHECKLIST

| # | Requirement | Done? |
| --- | --- | --- |
| 1 | Two routers with WAN 10.10.10.0/30 | [ ] |
| 2 | Static routing — all LANs reachable | [ ] |
| 3 | Admin LAN 192.168.10.0/24 — VLAN 10 | [ ] |
| 4 | Staff LAN 192.168.20.0/25 — VLAN 20 | [ ] |
| 5 | Wireless LAN 192.168.30.0/26 — VLAN 30 | [ ] |
| 6 | Server with HTTP, HTTPS, FTP enabled | [ ] |
| 7 | ACL: Only Admin can FTP | [ ] |
| 8 | ACL: Wireless cannot reach Admin LAN | [ ] |
| 9 | ACL: Block external attackers | [ ] |
| 10 | Port security max 1 MAC, violation demo'd | [ ] |
| 11 | Unused ports disabled in VLAN 99 | [ ] |
| 12 | Hostname set on all devices | [ ] |
| 13 | Console password: ciscoconpa55 | [ ] |
| 14 | Enable secret: cicsoenpa55 (note: typo in exam = intentional) | [ ] |
| 15 | service password-encryption | [ ] |
| 16 | MOTD banner on all devices | [ ] |
| 17 | Wireless WPA2 configured | [ ] |
| 18 | All devices labeled in topology | [ ] |
| 19 | PKT file named with student ID | [ ] |

# 10. CRITICAL GOTCHAS

| Trap | Fix |
| --- | --- |
| ACL applied wrong direction | Use 'in' on LAN interfaces, 'out' on WAN — wrong direction = ACL does nothing |
| Enable secret typo | Exam says 'cicsoenpa55' not 'ciscoenpa55' — type it exactly as given |
| Wildcard mask confusion | /24 = 0.0.0.255, /25 = 0.0.0.127, /26 = 0.0.0.63 |
| ACL implicit deny | Always add 'permit ip any any' at bottom or your server will block everything |
| Port security on trunk | NEVER apply port-security to trunk ports — only access ports |
| Unused VLAN not created | Create VLAN 99 first before assigning ports to it |
| Server services off | In Packet Tracer go to Server > Services tab and toggle HTTP/FTP ON |
| PC default gateway missing | Every PC needs a default gateway set or inter-VLAN routing fails |